# VALIDATION REPORT – VERSION 9

WFRC / MAG

September 14, 2023

Table of Contents

1	Household Disaggregation and Auto Ownership	1

2	Trip Generation	2

3	Trip Distribution	3

4	Mode Choice	4

4.1	Transit Trips and Boardings	4

4.2	Mode Share	7

5	Highway Assignment	11

5.1	Summary Comparison	11

5.2	Detailed Comparison	17

5.3	Map Comparison	23

List of Figures

Figure 4.1 Transit Trips by Hierarchical Mode - Model vs. Observed Comparison	5

Figure 4.2 Boardings by Hierarchical Mode - Model vs. Observed Comparison	5

Figure 4.3 Transfer Ratio by Hierarchical Model - Model vs. Observed Comparison	6

Figure 4.4 Boardings by Mode Surveyed - Model vs. Observed Comparison	6

Figure 4.5 Model vs Observed Mode Shares by Motorized / Non-Motorized (Daily, All Purposes)	8

Figure 4.6 Model vs Observed Mode Shares by Auto / Transit (Daily, All Purposes)	8

Figure 4.7 Model vs Observed Mode Shares by Drive Alone / Shared Ride (Daily, All Purposes)	9

Figure 4.8 Model vs Observed Mode Shares by Shared Ride # of Occupants (Daily, All Purposes)	9

Figure 4.9 Model vs Observed Mode Shares by Transit Mode (Daily, All Purposes)	9

Figure 4.10 Model vs Observed Mode Shares by Transit Access Mode (Daily, All Purposes)	10

Figure 5.1 Model vs Observed Percent Difference of All Roadways by Vehicle Type	12

Figure 5.2 Model vs Observed Percent Difference of Freeways by Vehicle Type	13

Figure 5.3 Model vs Observed Percent Difference of Principal Arterials by Vehicle Type	14

Figure 5.4 Model vs Observed Percent Difference of Minor Arterials by Vehicle Type	15

Figure 5.5 Model vs Observed Percent Difference of Collectors by Vehicle Type	16

Figure 5.6 Model vs Observed Volume and VMT Comparison (Region, All Vehicles)	18

Figure 5.7 Model vs Observed Volume and VMT Comparison (Box Elder - WFRC, All Vehicles)	19

Figure 5.8 Model vs Observed Volume and VMT Comparison (Weber - WFRC, All Vehicles)	20

Figure 5.9 Model vs Observed Volume and VMT Comparison (Davis, All Vehicles)	21

Figure 5.10 Model vs Observed Volume and VMT Comparison (Salt Lake, All Vehicles)	22

Figure 5.11 Model vs Observed Volume and VMT Comparison (Utah, All Vehicles)	23

Figure 5.12 Segment-Level Model vs Observed Volume Comparison by Vehicle Type	24

List of Tables

Table 1.1: Renumbered TAZ Ranges	18

# Trip Generation

# Trip Distribution

# Mode Choice

The validation results for the Mode Choice portion of the model are shown in this section. Mode Choice was validated against the Utah Transit Authority 2019 On-Board Survey as well as the 2012 Household Travel Survey. Validation is summarized by the following categories:

Transit Trips and Boardings

Mode Share

## Transit Trips and Boardings

The validation of daily transit trips and boardings is shown through the comparison of model and observed data by mode. The model was validated by the following measures:

Trips by Hierarchical Mode

Boardings by Hierarchical Mode

Transfer Ratio by Hierarchical Mode

Boardings by Surveyed Mode (for comparison only)

The three hierarchical mode measures are summarized by the highest hierarchy mode in a given trip with local bus being the lowest on the hierarchy and commuter rail being the highest. For example, if a trip uses Local Bus and then transfers to LRT, the trip is stored as an LRT trip. Trips by Hierarchical Mode represent each trip as a single number, regardless of the number of transfers or different modes that were used on a trip.

Boardings by Hierarchical Mode represent each boarding separately but summarized at the highest hierarchical mode of the trip. For example, for one transfer from Local Bus to LRT there are two boardings, one on each mode, but they are both stored in the highest hierarchal mode of LRT. The Transfer Ratio by Hierarchical Mode is the ratio between boardings and trips for any given mode.

The final measure Boardings by Surveyed Mode represents the total boardings on each mode individually. They are not summarized at the highest hierarchical mode of the trip but rather at the mode on which the boarding actually occurred. For example, the Local Bus to LRT trip mentioned previously would be summarized in this measure as one boarding on Local Bus and one boarding one LRT. This measure is for comparison use only since the structure of the Mode Choice model does not consider this measure during calibration.

The total number of boardings are the same between hierarchical mode and surveyed mode, but depending on the make-up of the trips, their totals by mode will vary.

The model data for hierarchical mode measures is taken from the shares reports v9_SE19_Net19_RegionShares_Pk.csv and v9_SE19_Net19_RegionShares_Ok.csv. The model data for the surveyed mode comparison is taken from _v9_SE19_Net19_1_PA_Route.dbf.

Figure 4.1, Figure 4.2, Figure 4.3, and Figure 4.4 show a comparison of model and observed values for Trips by Hierarchical Mode, Boardings by Hierarchical Mode, Transfer Ratio by Hierarchical Mode, and Boardings by Surveyed Mode respectively.

Figure 4.1 Transit Trips by Hierarchical Mode - Model vs. Observed Comparison

Figure 4.2 Boardings by Hierarchical Mode - Model vs. Observed Comparison

Figure 4.3 Transfer Ratio by Hierarchical Model - Model vs. Observed Comparison

Figure 4.4 Boardings by Mode Surveyed - Model vs. Observed Comparison

When considering Trips by Hierarchical Mode (Figure 4.1), the calibration routine of the model results in the total number of transit trips in the model being very close to the observed. For this model the model trips are only 0.2% higher than observed for the total number of trips. By mode the differences are within reasonable ranges for the higher trip modes. For example, LRT is only 0.1% high and CRT is only 0.8% low.

Due to the very low number of trips for Core Bus and Express Bus, their differences are acceptable. Currently, Core Bus for example has only one route. In future phases of the RTP, additional Core Bus service is planned. While further calibration could have brought the Core Bus validation more in line with observed, over-calibrating the model was not desired.

Boardings by Hierarchical Mode (Figure 4.2) show a similar pattern. Overall boardings are only 1.9% lower in model compared to observed, and the modes Local Bus, LRT, and CRT all within 5%. Core Bus is again the furthest away, but it also is a single route with lower number of boardings as compared to the higher boarding modes.

The Transfer Ratio by Hierarchical Mode (Figure 4.3) shows the relationship between the trips and the boardings. While the model was only slightly higher in trips, the boardings were low. This results in a lower transfer ratio, which is still acceptable with all values being within about 5%.

And for comparison, Boardings by Mode Surveyed (Figure 4.4) shows acceptable differences. The Core Bus and Express Bus values show much larger differences; but again given their low volumes, these are reasonable differences.

## Mode Share

Mode share validation is shown through the comparison by groups of mode, time period, and trip purpose. For model and observed shares, the total percent for each combination of mode group, time period, and trip purpose adds up to 100%.

Mode groups compared include groupings of auto, transit, vehicle occupancy, and transit modes. The highest nest is motorized and non-motorized down to lower nests of individual transit modes and transit access modes. Shares were calculated for the following mode groupings:

Motorized / Non-Motorized

Auto / Transit

Drive Alone / Shared Ride

Shared Ride # of Occupants

Transit Mode

Transit Access Mode

Additionally, the shares were compared by the following time period groupings:

Daily

Peak

Off-peak

Finally, the shares were further compared for each of the following trip purposes:

All Purposes

Home-Based Work

Home-Based College

Home-Based Other

Non-Home Based

In the model, Home-Based College is the only trip purpose that does not include trips in the off-peak time period. So, any trips with this purpose in the observed data were moved to the peak time period for comparison.

Figure 4.5, Figure 4.6, Figure 4.7, Figure 4.8, Figure 4.9, and Figure 4.10 compare model vs observed mode shares by specific mode groups for the daily time period for all purposes. The observed data in the figures comes from a combination of the Household Travel Survey and the 2019 Transit On-Board Survey. This document does not include figures for peak and off-peak time period comparisons nor for home-based work, home-based college, home-based other, and non-home based purpose comparisons (for these specific comparisons, please direct yourself to the online documentation).

Figure 4.5 Model vs Observed Mode Shares by Motorized / Non-Motorized (Daily, All Purposes)

Figure 4.6 Model vs Observed Mode Shares by Auto / Transit (Daily, All Purposes)

Figure 4.7 Model vs Observed Mode Shares by Drive Alone / Shared Ride (Daily, All Purposes)

Figure 4.8 Model vs Observed Mode Shares by Shared Ride # of Occupants (Daily, All Purposes)

Figure 4.9 Model vs Observed Mode Shares by Transit Mode (Daily, All Purposes)

Figure 4.10 Model vs Observed Mode Shares by Transit Access Mode (Daily, All Purposes)

The Motorized / Non-motorized grouping of model shares (Figure 4.5) are within 0.1% of the observed data for the daily time period and all trip purposes with non-motorized shares being slightly higher than observed and motorized shares being slightly lower. The model shares for additional combinations of time period and purpose are all within 0.5%.

The Transit Mode grouping of model shares (Figure 4.9) are also close to observed with all modes for all time periods and trip purposes. At the Daily time period for all purposes, Local Bus has slightly higher share at 1.7% compared to observed than other modes, such as CRT that is 0.5% lower than observed and LRT that is 0.3% lower than observed. Nearly all the model shares for Transit Mode combinations of time period and are within 5% of observed. The one exception is Home-Based Other which in the peak period for Local Bus is 5.8% higher than observed. Transit Access Mode model shares are all within 5% of observed shares.

# Highway Assignment

The validation results for the Highway Assignment portion of the model are shown in this section. The observed data for 2019 volumes is taken from the Utah Department of Transportation (UDOT) Average Annual Daily Traffic (AADT) History and associated with their respective model segments. The traffic model data is taken from segment summary report for the 2019 base year model: v9_SE19_Net19_Summary_SEGID.csv. The results are divided into three sections:

Summary Comparison

Detailed Comparison

Map Comparison

## Summary Comparison

The summary comparison shows region and county-wide differences between model and observed for Average Daily Volume and Vehicle-Miles Traveled (VMT) by vehicle type. The values for Box Elder and Weber counties are only the portions within the MPO planning area. Validation was checked comparing the average daily volume at the region and county levels. Figure 5.1, Figure 5.2, Figure 5.3, Figure 5.4, and Figure 5.5, contain percent difference validation charts for All Roadways, Freeways, Principal Arterials, Minor Arterials, and Collectors by All Vehicles, Passenger Cars, Medium Trucks, and Heavy Trucks respectively (for comparisons dealing with absolute difference, please direct yourself to the online documentation).

Figure 5.1 Model vs Observed Percent Difference of All Roadways by Vehicle Type

Figure 5.2 Model vs Observed Percent Difference of Freeways by Vehicle Type

Figure 5.3 Model vs Observed Percent Difference of Principal Arterials by Vehicle Type

Figure 5.4 Model vs Observed Percent Difference of Minor Arterials by Vehicle Type

Figure 5.5 Model vs Observed Percent Difference of Collectors by Vehicle Type

At the region level model volume is 0.2% lower than observed volume. The four more urban counties (Weber, Davis, Salt Lake, and Davis) were all within 5% of observed volumes with Salt Lake County being the closest. Weber and Davis were slightly lower and Utah County was slightly higher. Box Elder County is more rural than the other counties. Box Elder model volumes are about 10% lower than observed. Time did not allow for further calibration of the volumes in Box Elder area to account for the larger differences.

One important observation at the Collector (Figure 5.1) and All Vehicles (Figure 5.5) level is that Utah County shows a much higher difference than the other counties. Upon further investigation of observed Collector volumes in Utah County, many roadway segments had very low volumes compared to what was expected. Utah County is one of the highest growth areas in the region. For this reason, we expect that the observed count data may be underrepresenting actual volumes. We also anticipate observed volumes in Utah County to improve in the near-term. Within the last several years, a large investment in continuous count station in Utah County has been made. The new counters will add additional information to generate observed volumes for all roadway segments.

The largest differences in model vs observed volumes occur in the Medium Truck and Heavy Truck vehicle types. A good amount of time was spent attempting to bring model truck volumes closer to observed. However, due to the limited data sources for truck information, further need to investigate observed truck volumes, and a desire to not over-calibrate the model, further calibration was stopped. Truck modeling remains a future priority for model improvement.

## Detailed Comparison

The model vs observed details in this section are presented by Volume and Vehicle-Miles Traveled (VMT) through the comparison of model and observed data facility type by region and by county. Figure 5.6, Figure 5.7, Figure 5.8, Figure 5.9, Figure 5.10, and Figure 5.11 show model and observed values for the region and each county for all vehicles (for comparisons for cars, medium trucks, and heavy trucks, please direct yourself to the online documentation). The comparisons are shown in four different types of charts and tables:

Average Daily Volume by Roadway Class (2a): The daily volume is averaged across all segments within their respective geography and vehicle type.

Total VMT by Roadway Class (2b): For each segment, the daily volume is multiplied by segment distance and then summed across all segments within their respective geography and vehicle type.

Model vs Count Segment Volume (2c): This is a scatter plot of segment daily volume with the x-axis as the observed volume and the y-axis as the model volume. The red line shows the location of where model and observed volumes are equal. The dashed blue line shows a least-squares linear regression. The further the blue line moved away from the red line, the further the model is from observed.

Segment Percent Error (2d): This is a scatter plot showing the amount of error (percent difference) between the observed volume and the model volume. The observed volume is the x-axis and the percent error is the y-axis. The red lines are a bounding box that shows the control target. As volume increases, it is expected that the percent error should decrease.

Figure 5.6 Model vs Observed Volume and VMT Comparison (Region, All Vehicles)

Figure 5.7 Model vs Observed Volume and VMT Comparison (Box Elder - WFRC, All Vehicles)

Figure 5.8 Model vs Observed Volume and VMT Comparison (Weber - WFRC, All Vehicles)

Figure 5.9 Model vs Observed Volume and VMT Comparison (Davis, All Vehicles)

Figure 5.10 Model vs Observed Volume and VMT Comparison (Salt Lake, All Vehicles)

Figure 5.11 Model vs Observed Volume and VMT Comparison (Utah, All Vehicles)

## Map Comparison

The maps in Figure 5.12 shows a comparison of segment level model vs observed volumes by vehicle types. Blue represents model lower than observed and red represent model volume higher than observed.

Figure 5.12 Segment-Level Model vs Observed Volume Comparison by Vehicle Type

Looking at the All Vehicles map, the model volumes are lower than observed for by more than 15,000 vehicles per day for the east side of I-215 and for I-15 through northern Utah County. Model volumes are higher than observed volumes by more than 15,000 vehicles for I-15 in southern Salt lake County and for I-15 in Utah County between Springville and Spanish Fork. When looking at these areas by vehicle type, the drop in Cars and Light Trucks are actual greater since the Medium Trucks and Heavy Trucks in these areas are greater in the model vs observed. Outside of these areas, the volume differences between model and observed are relatively minor.

The lower arterial model vs observed volumes of Heavy Trucks on 9000 South in Salt Lake County was further investigated. The Heavy Truck observed volume for this roadway seemed much higher than expected for this roadway. The lower volumes are likely due to the observed data and not anything in the model.

Table 1.1: Renumbered TAZ Ranges

| Parameter | v9 Value | v8 Value | Notes |
| --- | --- | --- | --- |
| UsedZones | 3629 | 2881 | Highest TAZ number used by model |
| BoxElderRange | 1-153 | 1-140 | Box Elder County Range |
| WeberRange | 154-581 | 141-423 | Weber County Range |
| DavisRange | 582-905 | 424-654 | Davis County Range |
| SLRange | 906-2216 | 655-1788 | Salt Lake County Range |
| UtahRange | 2217-3546 | 1789-2881 | Utah County Range |
| Dummyzones | 3547-3600 | 2882-3400 | Placeholder for future TAZ splits |
| Externalzones | 3601-3629 | 136-140, 421-423, 1782-1788, 2874-2881 | External zones |
| NorthBC | 3604-3606 | 138, 139, 140 | North Brigham City external zones |